# Twinning Engineering Programmes (TEP) &

# Thammasat English Programme of Engineering (TEPE)

Faculty of Engineering, Thammasat University

Experiment 4: Superposition Principle

Objectives:

- To investigate the superposition principle (i.e. understand how the voltage across (or the current through) any element in the multi-independent source circuit can be obtained using the superposition principle).

Experimental Setup & Procedure

R5=500

R2=1k

R4=500

R3=1k

R1=1k2

B

D

C

A

+10V-

Figure 1

-15V+

- Set up the circuit in Figure 1.

- Measure the currents flowing through R1, R3 and R4 (Note the direction of the currents).

- Measure the voltage across the resistors R1, R3 and R4 (Note the polarities of the voltages).

- Disconnect the voltage source at C, then link points C and D together.

- Repeat step 2 and 3.

- Remove the link between points C and D and replace it with the +15V source.

- Disconnect the voltage source at A, then link points A and D together.

- Repeat step 2 and 3.

Question 1: Copy Figure 1 onto your report and mark the values and directions of the currents and voltages measured in steps 1 and 2.

Question 2: Copy Figure 1 onto your report and modify it accordingly to step 4.

Question 3: Mark the values and directions of the currents and voltages measured in step 5 in the new circuit obtained in Question 2.

Question 4: Copy Figure 1 onto your report and modify it accordingly to step 7.

Question 5: Mark the values and directions of the currents and voltages measured in step 8 in the new circuit obtained in Question 4.

Question 6: Use the superposition principle to calculate the currents flowing through R1, R3, R4. Compare the calculated values to the measured ones (Remember to show your calculation).

Question 7: Use the superposition principle to calculate the voltage across R1, R3, R4. Compare the calculated values to the measured ones (Remember to show your calculation).